SmartBranch360

Network Design Document

Secure Branch Office Network — Design, Security & Troubleshooting Reference

Submitted by: Deepesh Nayak

College: Medi-caps University, Indore

Course: B.Tech CSE (AI Specialization), Batch 2023–2027

AICTE Internship Student Registration ID : STU69d54005c39db1775583237

Technology: Networking

Submission Date: 30 August 2026

This project was completed individually by the above-mentioned student.

# 1. Project Overview

SmartBranch360 is a secure branch-office network that includes wired employees, guest Wi-Fi, an internal server, internet access, and secure device management. The network was designed and built in Cisco Packet Tracer using VLAN segmentation, routing, DHCP, NAT, and ACLs.

This document covers: the topology diagram, VLAN/IP address plan, security rules, normal-operation test results, and 5 fault-injection scenarios along with their fixes.

# 2. Network Topology

The diagram below shows the complete SmartBranch360 topology — BranchRouter (2911) connects to the internet cloud, Switch1 is the distribution layer connecting Switch2 (access), AP-Guest, Server1, and Mgmt-PC, and Switch2 connects the 8 employee PCs.

Figure: SmartBranch360 complete network topology — showing Cloud0 (ISP), BranchRouter, Switch1 & Switch2, AP-Guest with guest devices, Server1, Mgmt-PC, and all Employee PCs (Emp-PC1–Emp-PC8).

## 2.1 Device Roles

| Device | Role |
| --- | --- |
| Cloud0 (Cloud-PT) | Simulated ISP / internet connection |
| BranchRouter (2911) | Edge router — routing between VLANs, DHCP, NAT, ACLs |
| Switch1 (2960-24TT) | Distribution switch — trunks to router, Switch2, AP, Server, Mgmt-PC |
| Switch2 (2960-24TT) | Access switch — connects 8 employee PCs on VLAN 10 |
| AP-Guest | Wireless access point for Guest VLAN 20 (SSID: SmartBranch-Guest, WPA2-PSK) |
| Server1 | Internal server, static IP on VLAN 30 |
| Mgmt-PC | Management workstation, only device allowed to SSH into devices |
| Emp-PC1 – Emp-PC8 | Employee wired endpoints, VLAN 10, DHCP assigned |
| Guest-Laptop / Guest-Phone | Wireless guest endpoints, VLAN 20, DHCP assigned |

# 3. VLAN and IP Address Plan

| VLAN ID | Name | Subnet | Gateway |
| --- | --- | --- | --- |
| 10 | Employee | 192.168.10.0/24 | 192.168.10.1 |
| 20 | Guest | 192.168.20.0/24 | 192.168.20.1 |
| 30 | Server | 192.168.30.0/24 | 192.168.30.1 |
| 99 | Management | 192.168.99.0/24 | 192.168.99.1 |

## 3.1 Static Assignments

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Server1 | 192.168.30.10 | 255.255.255.0 | 192.168.30.1 |
| Mgmt-PC | 192.168.99.10 | 255.255.255.0 | 192.168.99.1 |

## 3.2 DHCP Pools

| Pool Name | Network | Default Router | DNS Server |
| --- | --- | --- | --- |
| EMPLOYEE_POOL | 192.168.10.0/24 | 192.168.10.1 | 8.8.8.8 |
| GUEST_POOL | 192.168.20.0/24 | 192.168.20.1 | 8.8.8.8 |

# 4. Security Rules

## 4.1 ACL 120 — Guest Isolation

The Guest VLAN (20) must not have access to the Server VLAN (30) — guests should only be able to use the internet, not reach the internal server. This ACL is bound inbound on the router's Gi0/1.20 sub-interface.

| Seq | Action | Source | Destination |
| --- | --- | --- | --- |
| 10 | deny | 192.168.20.0/24 | 192.168.30.0/24 |
| 20 | permit | any | any |

## 4.2 ACL 10 — Secure Management (SSH restriction)

SSH access to network devices is allowed only from the Management VLAN (99) — SSH is blocked from any employee or guest device. This is applied on the VTY lines.

| Seq | Action | Source |
| --- | --- | --- |
| 10 | permit | 192.168.99.0/24 |
| 20 | deny | any |

## 4.3 NAT Overload — Internet Access

Employee and Guest VLANs get internet access through NAT overload (PAT) via the router's WAN interface (Gi0/0).

ip nat inside source list 1 interface GigabitEthernet0/0 overload

# 5. Normal Operation — Test Results

| Test | Result |
| --- | --- |
| Employee → Server (ping) | PASS — reachable |
| Employee → Internet (NAT translation verified) | PASS — translation entries confirmed via 'show ip nat translations' |
| Guest → Server (ping) | PASS (blocked as expected) — 'Destination host unreachable' due to ACL 120 |
| SSH restriction test | PASS — Mgmt-PC could SSH in; Emp-PC1 was refused |

(1) successful ping from an Employee PC to the Server

(2) show ip nat translations' output

(3) failed ping from Guest to Server

(4) successful SSH login from Mgmt-PC vs refused SSH from an Employee PC.

# 6. Fault Scenarios & Troubleshooting Notes

5 faults were deliberately injected, verified, and fixed. Each fault's root cause and fix is given below.

| # | Fault Injected | Symptom | Root Cause | Fix Applied |
| --- | --- | --- | --- | --- |
| 1 | VLAN 20 removed from Switch1 trunk | Guest total isolation — no connectivity at all | VLAN 20 was accidentally removed from the trunk port's allowed-VLAN list | Re-added VLAN 20 to the trunk allowed list |
| 2 | Wrong static gateway on Emp-PC2 | Server unreachable from Emp-PC2 only | An incorrect default gateway IP was manually set on the PC | Set the correct gateway (192.168.10.1) |
| 3 | Extra ACL rule added on wrong interface | No visible effect — traffic continued normally | The ACL rule was created, but it wasn't bound to the interface where it needed to apply | Removed the extra/unbound rule |
| 4 | NAT overload rule removed | Internet access fails — no NAT translation happening | The 'ip nat inside source list 1 ... overload' command was accidentally deleted | Reconfigured the NAT overload rule |
| 5 | DHCP pool gateway changed | Server unreachable for DHCP clients (wrong gateway handed out) | The DHCP pool's default-router value was set incorrectly | Reverted the pool's default-router to the original gateway |

Fault #1 verification — Fixed. show interfaces trunk output confirms VLAN 20 is present on both Gig0/1 and Gig0/2 trunk ports, restoring Guest VLAN connectivity across the trunk.

Fault #2 verification — Fixed. Emp-PC2's IP Configuration confirms the Default Gateway is correctly set to 192.168.10.1, restoring connectivity to Server1 and other VLANs.

Fault #3 verification — Fixed. show access-lists 120 confirms only the intended 2 rules exist (deny Guest→Server, permit any any) — the earlier extra unbound rule has been removed.

Fault #4 verification — Fixed. show run | section nat confirms the NAT overload rule is present, restoring internet access for Employee and Guest VLANs

Fault #5 verification — Fixed. show run | section dhcp confirms both DHCP pools hand out the correct default-router (192.168.10.1 for Employee, 192.168.20.1 for Guest).

# 7. Python Assurance Tool (Reference)

A Python script (checker.py) was built to validate VLAN trunking, NAT, DHCP, ACLs (including interface binding), and static PC IPs against requirement.yaml (the source-of-truth design). Each finding is printed as PASS / WARN / FAIL along with a symptom and a suggested fix.

The full script, requirement.yaml, sample data, and sample validation report are submitted separately in the 'SmartBranch360_PythonChecker.docx' file (a separate deliverable).

Figure: Python Checker Tool run showing SmartBranch360 validation report — all VLANs, NAT, DHCP, ACLs, and static IP checks passed successfully (12/12).

# 8. Conclusion

The SmartBranch360 network was successfully designed, built, and tested — with VLAN segmentation, secure inter-VLAN routing, guest isolation, restricted management access, and internet connectivity. Troubleshooting was practiced by injecting 5 fault scenarios, and a Python-based assurance tool was built to automatically detect future configuration drift.